# PANDUAN PEMBUATAN
SMART GREENHOUSE IoT

Step-by-Step dari Nol sampai Jadi

ESP32-S3-N16R8 Soldered

Panduan ini dirancang untuk orang awam.

Ikuti step-by-step dari atas ke bawah.

Jangan loncat step — setiap step bergantung pada step sebelumnya.

13525004@std.stei.itb.ac.id | April 2026

# DAFTAR ISI

FASE 0: Persiapan Software (Step 1-3)

FASE 1: Rakit Hardware (Step 4-10)

FASE 2: Upload & Test Firmware (Step 11-15)

FASE 3: Setup Supabase (Step 16-19)

FASE 4: Test Sistem Lengkap (Step 20-24)

FASE 5: Web Dashboard (Step 25-27)

# FASE 0: Persiapan Software

Sebelum menyentuh hardware, siapkan software dulu.

## Step 1: Download & Install Arduino IDE

1. Buka browser, pergi ke arduino.cc/en/software

2. Download Arduino IDE 2.x (pilih sesuai OS: Windows/Mac/Linux)

3. Install seperti biasa (Next > Next > Finish)

4. Buka Arduino IDE setelah install selesai

## Step 2: Install ESP32 Board Support

Arduino IDE default hanya kenal Arduino Uno. Kita perlu menambahkan ESP32.

1. Buka Arduino IDE

2. File > Preferences

3. Di "Additional Board Manager URLs", paste:

https://espressif.github.io/arduino-esp32/package_esp32_index.json

4. OK > Tools > Board > Board Manager

5. Cari "esp32" > "esp32 by Espressif Systems" > Install

6. Tunggu selesai (5-10 menit)

## Step 3: Install Library yang Dibutuhkan

Library = kode tambahan supaya ESP32 bisa berkomunikasi dengan sensor.

Tools > Manage Libraries, cari & install:

a. "BH1750" by Christopher Laws

b. "DHT sensor library" by Adafruit

c. "Adafruit Unified Sensor" by Adafruit

d. "ESP32Servo" by Kevin Harrington

e. "ArduinoJson" by Benoit Blanchon

# FASE 1: Rakit Hardware

Rakit rangkaian elektronik di breadboard.

PERINGATAN: Jangan colok adaptor/USB sampai semua wiring selesai dan dicek!

## Step 4: Pasang ESP32 di Breadboard

1. Ambil breadboard 830 point

2. Tancapkan ESP32-S3 di TENGAH breadboard

3. Kedua sisi pin harus bisa diakses untuk jumper wire

TIP: Tekan kuat tapi hati-hati. Semua pin harus masuk.

## Step 5: Sambung Power Supply

1. Adaptor 5V 3A > jack DC female screw terminal (belum colok!)

2. Terminal (+) > power rail (+) breadboard = +5V

3. Terminal (-) > ground rail (-) breadboard = GND

4. Power rail (+) > ESP32 pin VIN

5. Ground rail (-) > ESP32 pin GND

6. ESP32 pin 3.3V > baris baru breadboard (jadi 3.3V bus sensor)

PERINGATAN: VIN ESP32 = 5V. Sensor = 3.3V. Jangan tukar!

## Step 6: Sambung Sensor Cahaya (BH1750)

BH1750 pakai I2C (SDA + SCL):

VCC  -->  3.3V bus

GND  -->  GND bus

SDA  -->  GPIO8

SCL  -->  GPIO9

ADDR -->  kosong

## Step 7: Sambung Sensor Suhu (DHT11)

DHT11 module 3-pin:

VCC  -->  3.3V bus

GND  -->  GND bus

DATA -->  GPIO4

TIP: Beli yang MODULE 3-pin (ada PCB), bukan komponen telanjang 4-pin.

## Step 8: Sambung Sensor Hujan (Rain Sensor)

Dari control board LM393:

VCC  -->  3.3V bus

GND  -->  GND bus

DO   -->  GPIO5

AO   -->  JANGAN DISAMBUNG!

PERINGATAN: Pin AO bisa output 5V. ESP32 ADC max 3.3V. Disambung = RUSAK PERMANEN.

## Step 9: Sambung Servo Motor (MG996R)

MERAH (VCC)   -->  +5V Rail (ADAPTOR!)

COKLAT (GND)  -->  GND bus

ORANYE (Signal) -->  GPIO13

Capacitor 470uF dekat servo:

Kaki panjang (+) --> +5V Rail

Kaki pendek (-) --> GND Rail

PERINGATAN: Servo power WAJIB dari adaptor. BUKAN ESP32! Servo butuh 2.5A, ESP32 max 500mA.

## Step 10: Sambung LED Warning + Resistor

GPIO2 > Resistor 220 Ohm > LED Merah > GND

Resistor: tidak ada polaritas. LED: kaki panjang = (+).

# FASE 2: Upload & Test Firmware

## Step 11: Checklist Sebelum Power On

Cek SEMUA item sebelum colok adaptor!

Setelah yakin:

1. Colok adaptor > cek LED power ESP32 menyala

2. Tunggu 10 detik > sentuh komponen

3. Kalau ada yang PANAS > CABUT SEGERA

## Step 12: Colok ESP32 ke Laptop via USB

1. Kabel USB-C DATA (bukan charge-only!)

2. Colok ke ESP32 dan laptop

3. Device Manager harus muncul COM port baru

Tidak muncul? Ganti kabel USB atau install driver CH340/CP2102.

## Step 13: Konfigurasi Arduino IDE

Board: ESP32S3 Dev Module

PSRAM: OPI PSRAM

Flash Size: 16MB (128Mb)

USB CDC On Boot: Enabled

Upload Speed: 921600

Port: COM yang muncul

PERINGATAN: PSRAM = OPI & USB CDC = Enabled WAJIB BENAR!

## Step 14: Siapkan File Firmware

1. Clone/download folder firmware/ dari repo

2. Buka smart_greenhouse.ino di Arduino IDE

3. Copy credentials.h.example > credentials.h

4. Edit credentials.h:

#define WIFI_SSID     "nama_wifi"

#define WIFI_PASSWORD "password_wifi"

#define SUPABASE_URL  "https://xxx.supabase.co"

#define SUPABASE_KEY  "eyJ..."

TIP: Isi WiFi dulu. Supabase nanti. Sistem tetap jalan offline.

## Step 15: Upload Firmware ke ESP32

Klik Upload > tunggu compile > tunggu upload.

Kalau gagal "Connecting...": tahan BOOT > tekan RESET > lepas > upload > lepas BOOT

## Step 16: Buka Serial Monitor

Tools > Serial Monitor, baud rate 115200.

Tekan RESET di ESP32. Harus muncul data sensor.

Kalau ada -1 = sensor error, cek kabelnya.

# FASE 3: Setup Supabase

## Step 17: Buat Akun & Project Supabase

supabase.com > Sign Up > New Project > smart-greenhouse

Region: Southeast Asia (Singapore)

## Step 18: Catat URL dan API Key

Settings > API:

Project URL: https://xxxxx.supabase.co

anon public key: eyJ...

Update credentials.h > upload ulang firmware.

PERINGATAN: Pakai "anon public" BUKAN "service_role" key!

## Step 19: Buat Tabel Database

SQL Editor > New Query > paste & Run:

CREATE TABLE sensor_logs (
  id BIGSERIAL PRIMARY KEY, temperature FLOAT, humidity FLOAT,
  lux FLOAT, is_raining BOOLEAN DEFAULT FALSE,
  vent_position INTEGER DEFAULT 0,
  roof_angle INTEGER DEFAULT 0,
  mode TEXT DEFAULT 'auto', overheating BOOLEAN DEFAULT FALSE,
  created_at TIMESTAMPTZ DEFAULT NOW()
);
CREATE TABLE commands (
  id BIGSERIAL PRIMARY KEY,
  vent_position INTEGER CHECK (vent_position BETWEEN 0 AND 100),
  mode TEXT DEFAULT 'auto',
  executed BOOLEAN DEFAULT FALSE,
  created_at TIMESTAMPTZ DEFAULT NOW(), executed_at TIMESTAMPTZ
);
ALTER TABLE sensor_logs ENABLE ROW LEVEL SECURITY;
ALTER TABLE commands ENABLE ROW LEVEL SECURITY;
CREATE POLICY "read" ON sensor_logs FOR SELECT USING (true);
CREATE POLICY "insert" ON sensor_logs FOR INSERT WITH CHECK (true);
CREATE POLICY "all" ON commands FOR ALL USING (true) WITH CHECK (true);

## Step 20: Verifikasi Data Masuk

Tunggu 60 detik > Table Editor > sensor_logs > harus ada data baru!

# FASE 4: Test Sistem Lengkap

## Step 21: Test: Sensor Baca Normal

Serial Monitor tiap 5 detik: suhu (20-35C), humidity (40-90%), cahaya (0-65000 lux).

## Step 22: Test: Hujan = Atap Tutup (Rule 1)

Teteskan air ke rain plate > "Hujan: YA" > tunggu 60 detik > servo ke posisi 0% (0°).

Rule 1: Local is King. Tidak bisa di-override.

## Step 23: Test: Cerah = Atap Buka

Keringkan rain plate > arahkan senter ke BH1750 > tunggu 60 detik > servo ke posisi 100% (180°).

## Step 24: Test: Hysteresis

Tetes air > tunggu 30 detik > keringkan > servo TIDAK BOLEH bergerak!

## Step 25: Test: Anti-Oven (Rule 4)

Hujan aktif + suhu > 33C: LED berkedip, atap tetap tutup.

# FASE 5: Web Dashboard (Opsional)

## Step 26: Setup Next.js

npx create-next-app@latest greenhouse-dashboard

npm install @supabase/supabase-js

.env.local: NEXT_PUBLIC_SUPABASE_URL & NEXT_PUBLIC_SUPABASE_ANON_KEY

## Step 27: Deploy ke Vercel

Push ke GitHub > vercel.com > Import > Add env vars > Deploy

## Step 28: Test Dashboard

Data sensor muncul, slider posisi ventilasi 0-100% berfungsi, tombol AUTO/MANUAL aktif.

# SISTEM LENGKAP

Selamat! Smart Greenhouse IoT sudah berjalan.

Sensor tiap 5s | Supabase tiap 60s | Command tiap 10s | Weather tiap 30min | Servo 0-100% | Offline OK

13525004@std.stei.itb.ac.id | Smart Greenhouse IoT | April 2026